Basic Networking Interview Questions

1. What is a Computer Network?

A computer network is a collection of interconnected devices that communicate and share resources like data, printers, internet, etc.

2. What are the types of networks?

- LAN – Local Area Network (small area, office/home)

- MAN – Metropolitan Area Network (city-wide)

- WAN – Wide Area Network (large area, internet)

3. What is the OSI Model?

OSI has 7 Layers:

- Physical Layer

- Data Link Layer

- Network Layer

- Transport Layer

- Session Layer

- Presentation Layer

- Application Layer

Mnemonic:
Please Do Not Throw Sausage Pizza Away

4. Difference between OSI and TCP/IP Model

| OSI | TCP/IP |
| --- | --- |
| 7 Layers | 4 Layers |
| Conceptual model | Practical model |
| Reference model | Protocol suite |

TCP/IP Layers:

- Network Access

- Internet

- Transport

- Application

5. What is IP Address?

Unique address assigned to a device in network.

Example:

- IPv4: 192.168.1.1

- IPv6: 2001:db8::1

6. Difference between IPv4 and IPv6

| IPv4 | IPv6 |
| --- | --- |
| 32-bit | 128-bit |
| Uses dots | Uses colons |
| Limited addresses | Huge address space |

7. What is MAC Address?

Physical address of Network Interface Card (NIC).
Example:
00:1A:2B:3C:4D:5E

8. What is DNS?

Domain Name System converts website names into IP addresses.

Example:
google.com → IP Address

9. What is DHCP?

Dynamic Host Configuration Protocol automatically assigns:

- IP Address

- Subnet Mask

- Gateway

- DNS

10. What is Subnet Mask?

Divides network and host portion.

Example:
255.255.255.0

11. What is Default Gateway?

Router IP used to communicate outside local network.

12. What is TCP vs UDP?

| TCP | UDP |
| --- | --- |
| Connection oriented | Connectionless |
| Reliable | Fast but unreliable |
| Acknowledgment | No acknowledgment |

Examples:

- TCP → HTTP, FTP

- UDP → Video streaming, Gaming

13. What is Three-Way Handshake?

TCP establishes connection using:

- SYN

- SYN-ACK

- ACK

14. What is a Port?

Logical communication endpoint.

Examples:

- 80 → HTTP

- 443 → HTTPS

- 21 → FTP

- 22 → SSH

15. What is a Protocol?

Rules governing communication.

Examples:

- HTTP

- FTP

- SMTP

- TCP/IP

Devices Questions

16. Difference Between Hub, Switch and Router

| Device | Function |
| --- | --- |
| Hub | Broadcasts to all |
| Switch | Uses MAC address |
| Router | Uses IP routing |

17. What is Router?

Routes packets between networks.

18. What is Switch?

Connects devices in LAN.

19. What is Modem?

Converts digital ↔ analog signals.

20. What is Firewall?

Security system that filters traffic.

Important Protocol Questions

21. What is HTTP and HTTPS?

- HTTP = HyperText Transfer Protocol

- HTTPS = Secure HTTP (SSL/TLS)

22. What is FTP?

File Transfer Protocol.

23. What is SMTP, POP3, IMAP?

Email protocols.

- SMTP → Sending mail

- POP3 → Download mail

- IMAP → Sync mail

24. What is ARP?

Address Resolution Protocol maps IP → MAC.

25. What is ICMP?

Used for diagnostics.

Example:
ping command

Security Questions

26. What is VPN?

Virtual Private Network for secure communication.

27. What is Encryption?

Converting data into secure unreadable form.

28. Difference between Public IP and Private IP?

Private:

- 192.168.x.x

- 10.x.x.x

Public:
Internet accessible IP

Practical Commands Interview Questions

29. What does PING do?

Checks connectivity.

ping google.com

30. What is traceroute / tracert?

Shows route packets travel.

tracert google.com

31. What is ipconfig?

Displays IP settings (Windows)

ipconfig

Fresher HR + Technical Mixed Questions

32. What happens when you type google.com in browser?

Expected flow:
DNS → IP Resolution → TCP Handshake → HTTP Request → Server Response

33. Explain Packet Switching

Data broken into packets and transmitted separately.

34. What is Bandwidth?

Maximum data transfer capacity.

35. What is Latency?

Delay in packet travel.

Common Viva Questions

- What is topology?

- Types of topology?

- Difference between unicast, multicast, broadcast?

- What is packet?

- What is gateway?

- What is NAT?

- What is proxy server?

- What is collision?

- What is VLAN?

Frequently Asked Interviewer Favorites

- Explain OSI layers.

- TCP vs UDP.

- What happens when you open a website?

- Difference between switch and router.

- What is DNS?

- Explain subnet mask.

- What is ARP?

- What is ping?

- What is IP vs MAC address?

- Explain three-way handshake.

Mini MCQ Practice

Q1: Which layer does IP work on?
A) Transport
B) Network ✅
C) Session
D) Physical

Q2: Which protocol is connection-oriented?
A) UDP
B) TCP ✅

Q3: Port number for HTTPS?
A) 21
B) 80
C) 443 ✅

Q4: DNS stands for?
A) Dynamic Name System
B) Domain Name System ✅

Great — here are more Networking Interview Questions for BE Computer Engineering Freshers (interviewers often ask these to go beyond basics):

Advanced Fresher Networking Questions

36. What is Network Topology?

Types:

- Bus

- Star

- Ring

- Mesh

- Tree

Most used: Star topology

37. Difference between Unicast, Broadcast and Multicast

| Type | Meaning |
| --- | --- |
| Unicast | One to One |
| Broadcast | One to All |
| Multicast | One to Many |

38. What is NAT?

Network Address Translation

Converts private IP to public IP.

Example:
Many devices share one internet IP.

39. What is VLAN?

Virtual LAN divides one physical network into multiple logical networks.

Benefits:

- Security

- Reduced traffic

- Better management

40. What is Collision?

When two devices send data simultaneously.

Occurs in hubs.

41. What is CSMA/CD?

Carrier Sense Multiple Access/Collision Detection.

Used in Ethernet.

42. What is CSMA/CA?

Collision Avoidance.

Used in Wi-Fi.

Routing Questions

43. What is Routing?

Finding best path for packets.

44. What is Static Routing?

Manually configured routes.

45. What is Dynamic Routing?

Routes learned automatically.

Examples:

- OSPF

- RIP

- BGP

46. What is OSPF?

Open Shortest Path First.

Uses shortest path algorithm.

47. What is BGP?

Border Gateway Protocol.

Used on internet between ISPs.

48. What is RIP?

Routing Information Protocol.

Uses hop count.

TCP/IP Deep Questions

49. What is TCP Sliding Window?

Controls data flow between sender and receiver.

50. What is Flow Control?

Prevents receiver overload.

51. What is Congestion Control?

Controls excessive network traffic.

TCP methods:

- Slow Start

- Congestion Avoidance

52. What is Socket?

Combination of:
IP + Port

Used in network programming.

53. What is MTU?

Maximum Transmission Unit.

Maximum packet size.

Ethernet:
1500 bytes

54. What is TTL?

Time To Live.

Limits packet lifetime.

55. What is Fragmentation?

Breaking large packets into smaller packets.

Security Networking Questions

56. What is SSL/TLS?

Provides secure communication.

Used in HTTPS.

57. What is SSH?

Secure Shell.

Remote secure login.

Port 22

58. Difference between Symmetric and Asymmetric Encryption

| Symmetric | Asymmetric |
| --- | --- |
| One key | Two keys |
| Fast | More secure |

59. What is a Proxy Server?

Intermediate server between client and internet.

60. What is IDS and IPS?

IDS:
Intrusion Detection System

IPS:
Intrusion Prevention System

Wireless Networking Questions

61. What is Wi-Fi?

Wireless networking technology.

62. Difference between 2.4 GHz and 5 GHz

| 2.4 GHz | 5 GHz |
| --- | --- |
| Long range | Higher speed |
| More interference | Less interference |

63. What is SSID?

Wi-Fi network name.

64. What is Access Point?

Device providing wireless connectivity.

65. Difference between Bluetooth and Wi-Fi

| Bluetooth | Wi-Fi |
| --- | --- |
| Short range | Longer range |
| Low power | Higher speed |

Scenario Based Interview Questions

66. Ping works but website not opening. Why?

Possible reasons:

- DNS issue

- Browser issue

- Firewall

- Server down

67. Internet connected but slow. Reasons?

- Bandwidth issue

- Congestion

- Packet loss

- High latency

68. System gets IP 169.254.x.x — why?

APIPA address.

DHCP failed.

69. How do you troubleshoot no network?

Check:

- Cable

- IP Config

- Ping localhost

- Ping gateway

- Ping internet

- Check DNS

Important Port Numbers (Very Common)

Memorize these:

| Protocol | Port |
| --- | --- |
| HTTP | 80 |
| HTTPS | 443 |
| FTP | 21 |
| SSH | 22 |
| Telnet | 23 |
| SMTP | 25 |
| DNS | 53 |
| DHCP | 67/68 |

Tricky Interview Questions

70. Difference between Hub and Switch?

Hub broadcasts all.

Switch forwards intelligently using MAC.

71. Why TCP is reliable?

- ACK

- Retransmission

- Sequencing

72. What happens if subnet mask is wrong?

Communication failure.

73. Why use MAC if IP exists?

IP can change.

MAC is unique hardware address.

74. Why is ARP needed?

IP communication requires MAC.

ARP resolves it.

Freshers Viva Questions

- What is packet sniffing?

- What is packet loss?

- What is gateway vs router?

- Difference between LAN and VLAN?

- What is load balancing?

- What is QoS?

- What is DNS cache?

- What is DHCP lease?

- What is duplex mode?

- What is port forwarding?

Mini MCQs

75. Which protocol uses port 53?
A FTP
B DNS ✅
C HTTP
D SSH

76. Which device works at Network Layer?
A Hub
B Switch
C Router ✅

77. APIPA address starts with
A 10.x.x.x
B 169.254.x.x ✅

78. BGP is used for
A LAN
B Internet routing ✅

Complete Networking Interview Pack (BE Computer Engineering Fresher)

Part 1 — CCNA/Cisco Style Interview Questions

1. What is a Router?

A Layer 3 device that forwards packets based on IP addresses.

2. What is a Switch?

A Layer 2 device that forwards frames using MAC addresses.

3. Difference Between Router and Switch

| Switch | Router |
| --- | --- |
| Works on MAC | Works on IP |
| Layer 2 | Layer 3 |
| Within LAN | Connects networks |

4. What is Cisco IOS?

Internetwork Operating System used on Cisco devices.

5. What is a Default Gateway?

Router address used to reach external networks.

6. Common Cisco Commands

enable

configure terminal

show ip interface brief

show running-config

show version

copy run start

7. What is VLAN?

Virtual LAN logically segments networks.

Example:

- VLAN 10 HR

- VLAN 20 Finance

- VLAN 30 IT

8. What is Trunk Port?

Carries traffic for multiple VLANs.

9. Access Port vs Trunk Port

| Access | Trunk |
| --- | --- |
| One VLAN | Multiple VLANs |
| PC Connection | Switch-to-switch |

10. What is STP?

Spanning Tree Protocol.
Prevents loops.

11. What is Loop in Network?

Packets circulate endlessly causing broadcast storms.

12. What is Inter-VLAN Routing?

Allows communication between VLANs using router or Layer-3 switch.

13. What is Subnetting?

Dividing one network into smaller networks.

Example:
192.168.1.0/24
can become
192.168.1.0/26
192.168.1.64/26

14. What is CIDR?

Classless Inter Domain Routing.
Example:
/24
/16
/30

15. What is ACL?

Access Control List.
Filters traffic.

Types:

- Standard ACL

- Extended ACL

Part 2 — Troubleshooting Interview Round

16. User cannot access internet. Steps?

1 Check cable
2 Check IP address
3 Ping localhost
4 Ping gateway
5 Ping 8.8.8.8
6 Check DNS

17. Ping IP works but domain doesn't

Likely DNS issue.

18. Duplicate IP problem symptoms

- Intermittent disconnect

- IP conflict warning

19. High latency causes

- Congestion

- Packet loss

- Weak signal

20. Command Line Troubleshooting

ping

tracert

ipconfig /all

netstat

nslookup

arp -a

Part 3 — Frequently Asked Viva Questions

21 What is ARP?

22 What is RARP?

23 What is NAT?

24 PAT vs NAT?

25 What is DHCP Lease?

26 What is Broadcast Domain?

27 What is Collision Domain?

28 What is QoS?

29 What is MTU?

30 What is Port Forwarding?

Part 4 — 50 Networking MCQs

MCQ Set

1.

OSI has how many layers?
A 5
B 6
C 7 ✅
D 8

2.

IP works at:
A Physical
B Data Link
C Network ✅
D Transport

3.

Switch works on:
A IP
B MAC ✅
C Port
D DNS

4.

Router works at:
A Layer 1
B Layer 2
C Layer 3 ✅
D Layer 7

5.

Port of HTTPS
A 80
B 21
C 443 ✅
D 25

6.

DNS uses port:
A 22
B 53 ✅
C 80
D 443

7.

Which protocol is reliable?
A UDP
B TCP ✅

8.

Which is connectionless?
A TCP
B UDP ✅

9.

ARP maps:
A MAC to IP
B IP to MAC ✅

10.

APIPA starts with:
A 10.
B 172.
C 192.
D 169.254 ✅

11.

FTP Port:
21 ✅

12.

SSH Port:
22 ✅

13.

SMTP Port:
25 ✅

14.

Telnet Port:
23 ✅

15.

DHCP stands for?
Dynamic Host Configuration Protocol ✅

16.

Device connecting networks:
Router ✅

17.

Hub is:
Layer 1 ✅

18.

Switch is:
Layer 2 ✅

19.

BGP used for:
Internet routing ✅

20.

RIP metric:
Hop Count ✅

21.

Protocol for email sending:
SMTP ✅

22 POP3 receives mail

23 ICMP used by Ping

24 VLAN means Virtual LAN

25 OSPF means Open Shortest Path First

26 Full form LAN
Local Area Network

27 Full form WAN
Wide Area Network

28 Full form MAN
Metropolitan Area Network

29 IPv4 bits
32

30 IPv6 bits
128

31 MAC address is physical address

32 Firewall provides security

33 VPN means Virtual Private Network

34 HTTP is unsecured

35 HTTPS is secure

36 CSMA/CD used in Ethernet

37 CSMA/CA used in WiFi

38 Default subnet mask class C
255.255.255.0

39 Loop prevented by
STP

40 Packet loss means dropped packets

41 Which layer has TCP
Transport

42 Which layer has IP
Network

43 DNS converts names to IP

44 Traceroute shows path

45 Broadcast sends to all

46 Multicast sends to many

47 Unicast one to one

48 Router uses routing table

49 NAT saves public IPs

50 Gateway connects outside network

Part 5 — Scenario Based Interview Questions (Very Common)

Explain what happens when you open google.com?

Answer:
1 Browser checks cache
2 DNS resolution
3 TCP 3-way handshake
4 HTTP/HTTPS request
5 Server response
6 Page loads

What is difference between ping and traceroute?

Ping checks reachability.
Traceroute checks route.

Why use TCP over UDP?

Reliability.

If one system in LAN can't talk to others?

Check:

- IP

- Subnet mask

- Cable

- Switch port

- Firewall

Part 6 — HR + Networking Combined Questions

1 Tell me about OSI model.

2 Explain TCP handshake.

3 Explain DNS.

4 Explain subnetting.

5 Difference TCP and UDP.

6 What is packet switching?

7 Why networking interests you?

8 What is your final year networking project?

9 Difference hub switch router.

10 Explain VLAN.

Part 7 — Interviewer Favorite Tricky Questions

Q Why 127.0.0.1 used?
Loopback.

Q Why MAC and IP both needed?
Logical + Physical addressing.

Q Why switch better than hub?
Less collision.

Q Why subnetting used?
Security + efficient IP usage.

Q Why use VLAN?
Segmentation.

Mini Rapid Fire (Very Common)

What is:

- NAT

- PAT

- DHCP

- DNS

- VPN

- Firewall

- Proxy

- ACL

- STP

- OSPF

- BGP

- RIP

- VLAN

- MTU

- TTL

Prepare one-line answers.

Top Companies Fresher Networking Questions

TCS/Infosys

Mostly basics:

- OSI

- TCP UDP

- IP classes

- DNS

- Ports

Cognizant/Accenture

- Troubleshooting

- Networking commands

- Scenario questions

Cisco Support Roles

- VLAN

- Routing

- Subnetting

- STP

- ACL

Very Important Subnetting Questions Interviewers Ask

1 Find hosts in /24
254

2 Hosts in /30
2 usable

3 Hosts in /29
6 usable

4 Why /30 used?
Point-to-point links.

Must Memorize Ports

80 HTTP
443 HTTPS
20/21 FTP
22 SSH
23 Telnet
25 SMTP
53 DNS
67 DHCP Server
68 DHCP Client
110 POP3
143 IMAP

Bonus Practical Lab Questions

Configure IP in Cisco

conf t

interface fa0/1

ip address 192.168.1.1 255.255.255.0

no shutdown

Save config

copy running-config startup-config

Show interfaces

show ip interface brief

Final Interview Preparation Tip

Focus these 15 topics strongly:

1 OSI Model
2 TCP/IP
3 TCP vs UDP
4 IP Addressing
5 Subnetting
6 DNS
7 DHCP
8 VLAN
9 Routing
10 NAT
11 ARP
12 Ports
13 Troubleshooting
14 Cisco commands
15 Network security basics

If you master these, you can clear most fresher interviews.

Advanced Networking Interview Questions (Fresher → Higher Level)

1) Explain encapsulation and decapsulation.

- At sender:
Application Data → TCP Segment → IP Packet → Ethernet Frame → Bits

- At receiver:
Bits → Frame → Packet → Segment → Data

2) Explain packet vs frame vs segment.

- Segment → Transport layer (TCP)

- Packet → Network layer (IP)

- Frame → Data Link layer

Common interview favorite.

3) Difference between circuit switching and packet switching?

| Circuit Switching | Packet Switching |
| --- | --- |
| Dedicated path | No dedicated path |
| Telephone | Internet |
| Reliable | Efficient |

4) Explain TCP 3-Way Handshake

1 SYN
2 SYN-ACK
3 ACK

Connection established.

5) Explain TCP 4-way termination

FIN
ACK
FIN
ACK

6) What is sliding window protocol?

Controls flow of packets and acknowledgments.

7) What is Stop-and-Wait vs Sliding Window?

| Stop Wait | Sliding Window |
| --- | --- |
| One packet at a time | Multiple packets |
| Slow | Faster |

8) What is checksum?

Error detection mechanism.

9) Difference between error detection and correction?

- Detection → CRC, checksum

- Correction → Hamming code

10) Explain CRC.

Cyclic Redundancy Check detects transmission errors.

Advanced IP Questions

11) Explain CIDR notation.

192.168.1.0/24

/24 means 24 network bits.

12) What is subnetting?

Dividing network into smaller networks.

Possible question:
Find hosts in /27

Answer:
32 addresses
30 usable hosts

13) Difference between subnetting and supernetting.

Subnetting:
Split network.

Supernetting:
Combine networks.

14) What is NAT vs PAT?

NAT:
One private ↔ one public

PAT:
Many private share one public

15) Explain private IP ranges.

- 10.0.0.0/8

- 172.16–31

- 192.168.0.0/16

Routing Advanced

16) Distance Vector vs Link State

| Distance Vector | Link State |
| --- | --- |
| RIP | OSPF |
| Slow | Faster |
| Hop count | SPF algorithm |

17) What is Dijkstra Algorithm?

Used by OSPF for shortest path.

18) What is routing table?

Stores path information.

19) Static vs Dynamic Routing?

Classic interview.

20) What is default route?

0.0.0.0/0

Route of last resort.

Switching Advanced

21) What is Spanning Tree Protocol?

Prevents switching loops.

22) What is Broadcast Storm?

Excessive broadcasts flood network.

23) What is collision domain vs broadcast domain?

Switch:
Each port collision domain.

Router:
Breaks broadcast domains.

24) What is trunking?

Carries multiple VLAN traffic.

25) What is inter-VLAN routing?

Communication between VLANs.

Security Questions

26) Symmetric vs Asymmetric encryption

Examples:
Symmetric:

- AES

Asymmetric:

- RSA

27) What is SSL handshake?

May be asked in advanced interviews.

28) What is Man in the Middle attack?

Security concept question.

29) Difference between IDS and IPS?

Common security round.

30) What is firewall stateful inspection?

Advanced security question.

Protocol Deep Questions

31) How DNS resolution works?

Browser → Resolver → Root → TLD → Authoritative DNS

Very common advanced question.

32) Difference HTTP vs HTTPS?

Use TLS encryption.

33) What happens when you open google.com?

Classic interview favorite.

Full answer:

- DNS lookup

- ARP

- TCP handshake

- TLS (if HTTPS)

- HTTP request

- Response

34) Explain ARP process.

“How does one machine find another MAC?”

35) What is ARP Spoofing?

Advanced security question.

Troubleshooting Scenario Questions

36) Ping gateway works but internet doesn't.

Possible:

- DNS

- ISP

- Default route

37) Cannot ping by hostname but IP works?

DNS issue.

38) Duplicate IP issue symptoms?

Conflict.

39) Packet loss reasons?

- Congestion

- Errors

- Bad cable

40) Why latency happens?

- Distance

- Routing

- Queueing

Interviewer Tricky Questions

41) Why TCP reliable but UDP faster?

42) Why use UDP for video streaming?

43) Why does switch need MAC table?

44) Why ARP not use TCP?

45) Why TTL needed?

46) Why fragmentation happens?

47) Why MTU mismatch causes issue?

48) Explain SYN flood attack.

49) Why ports needed if IP exists?

50) Explain socket.

IP + Port

Advanced Fresher MCQs

Which protocol uses Dijkstra?
A RIP
B OSPF ✅

Default route is
A 127.0.0.1
B 0.0.0.0/0 ✅

Broadcast storm prevented by
A DHCP
B STP ✅

ARP maps
IP → MAC ✅

Which protocol unreliable but fast
UDP ✅

CCNA / CCNP Lab Interview Questions with Answers (Hands-On Round)

Subnetting Problems with Answers

1) How many usable hosts in /24?

Answer:
256 total
254 usable

2) Hosts in /30?

Answer:
4 total
2 usable
Used for point-to-point links.

3) Hosts in /29?

Answer:
8 total
6 usable.

4) Find Network ID

IP: 192.168.10.45/27
Answer:
Network ID: 192.168.10.32
Broadcast: 192.168.10.63
Hosts: .33 to .62

5) Divide 192.168.1.0/24 into 4 subnets

Answer:
/26 subnets:
192.168.1.0
192.168.1.64
192.168.1.128
192.168.1.192

Cisco Configuration Lab Questions

6) Configure IP on Router

Question:
Configure router interface with:
IP 192.168.1.1/24

Answer:

enable

conf t

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

7) Configure Static Route

Question:
Reach 10.0.0.0 via 192.168.1.2

Answer:

ip route 10.0.0.0 255.0.0.0 192.168.1.2

8) Default Route Configuration

ip route 0.0.0.0 0.0.0.0 192.168.1.254

VLAN Lab

9) Create VLAN 10

conf t

vlan 10

name SALES

10) Assign Port to VLAN

interface fa0/1

switchport mode access

switchport access vlan 10

11) Configure Trunk

interface fa0/24

switchport mode trunk

12) Verify VLANs

show vlan brief

OSPF Labs

13) Configure OSPF

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

14) Verify OSPF Neighbors

show ip ospf neighbor

15) OSPF Adjacency not forming — troubleshoot?

Check:

- Area mismatch

- Hello timer

- Authentication

- MTU mismatch

ACL Lab Questions

16) Standard ACL Example

Permit 192.168.1.0 network:

access-list 10 permit 192.168.1.0 0.0.0.255

17) Extended ACL Example

Block Telnet:

access-list 101 deny tcp any any eq 23

access-list 101 permit ip any any

NAT Lab

18) Configure NAT basic

ip nat inside

ip nat outside

19) PAT Example

ip nat inside source list 1 interface g0/0 overload

STP Labs

20) Make Switch Root Bridge

spanning-tree vlan 1 root primary

21) Enable PortFast

spanning-tree portfast

22) Enable BPDU Guard

spanning-tree bpduguard enable

Troubleshooting Lab Scenarios

23) Interface down/down means?

Possible:

- Cable disconnected

- Device off

24) Interface up/down means?

Layer 2 issue.
Encapsulation mismatch.

25) Interface errors increasing?

Check:

show interfaces

Possible CRC/duplex issue.

26) User can’t reach gateway

Check:

- VLAN

- IP

- Mask

- Port assignment

27) OSPF stuck EXSTART

Usually MTU mismatch.

28) Switch loop suspected

Use:

show spanning-tree

Packet Analysis Questions

29) How identify TCP handshake in packet capture?

SYN
SYN ACK
ACK

30) Identify retransmissions in Wireshark.

Repeated sequence numbers.

CCNP Style Tough Questions

31) Explain Route Redistribution Example.

Redistribute RIP into OSPF:

router ospf 1

redistribute rip subnets

32) Summarization Example

area 1 range 10.1.0.0 255.255.0.0

33) Floating Static Route?

Backup route with higher AD.

Example:

ip route 10.0.0.0 255.0.0.0 192.168.1.2 200

34) Verify Routing Table

show ip route

35) Show MAC Table

show mac address-table

Lab Interview Rapid Fire

- Difference running-config and startup-config?

- Command save configuration?

copy run start

- Show CDP neighbors?

show cdp neighbors

- Show LLDP neighbors?

show lldp neighbors

- Check trunk interfaces?

show interfaces trunk

Common Practical Interview Tasks

Task 1

Configure:

- 2 VLANs

- trunk

- inter-vlan routing

- verify connectivity

Task 2

Given 3 routers:
Configure OSPF adjacency.

Task 3

Given broken connectivity:
Troubleshoot.

Task 4

Configure ACL blocking Telnet but allow Ping.

Task 5

Configure PAT for internet access.

Interviewers May Give Debug Scenarios

- Why OSPF neighbor Full not forming?

- Why BGP idle state?

- Why VLAN hosts isolated?

- Why pings fail one direction?

- Why trunk not passing VLAN?

- Why routes not learned?

- Why interface shows CRC errors?

- Why packet drops under load?

- Why STP blocking unexpected port?

- Why DHCP clients not receiving IP?

Most Important Show Commands

Memorize:

show ip route

show ip interface brief

show interfaces

show vlan brief

show spanning-tree

show ip ospf neighbor

show mac address-table

show arp

show cdp neighbors

show running-config

Interview Favorite Subnet Problems

- Find broadcast for 172.16.10.34/28

- Design subnet for 100 hosts.

- Design subnet for 500 hosts.

- VLSM problem.

- Find first usable /27 subnet.

Bonus TAC Scenarios

- Packet loss only during peak hours — why?

- High CPU on router troubleshoot.

- Intermittent BGP flaps.

- One application slow but ping okay.

- VPN tunnel unstable.